Appendix 8 to the Grant Agreement

dated from " " ____________ 20___ No. ________

ENVIRONMENTAL MANAGEMENT PLAN (EMP)

GENERAL INFORMATION ABOUT THE PROJECT AND THE PROJECT SITE

- 1.1     DESCRIPTION OF THE PROJECT

- 1.1.1 Name of the project

"Development of a 500 kV shunt reactor with an integrated monitoring system and implementation in production with the purpose of commercializing the results of scientific and technical activities".

- 1.1.2 Location of the project

Production capacity created on the basis of “Asia Trafo” LLP (“Asia Trafo”) in the Republic of Kazakhstan, Shymkent city, Karatau area, residential area Tassay, Building 1196. The plant for the production of power transformers is located in the north-eastern part of Shymkent in the “Tassay” industrial zone.

The enterprise includes a complex of necessary buildings, structures and premises for industrial and domestic purposes, located on a land plot with an area of 12.0 hectares (cadastral number 19-309-227-1196). The site from the north-west and south-west of the plant borders on the enterprises of the Industrial zone "Tassay", from the north-east on the residential area Zhagalbayly, from the south-east on the highway Zhibek Zholy.

The nearest residential development from the emission sources is located:

- from the southeast - at a distance of 220 m;

- from the north-east - at a distance of 200 m.

Manufacturing takes place in workshops on the territory of the plant. The year of construction is 2018. External and internal walls of the building - metal caisson, inter-floor covering.

Objective of the project

- To carry out a set of measures taken for the development of a 500 kV shunt reactor with an integrated monitoring system and implementation in production in order to commercialize the results of scientific and technical activities within the framework of the "Productive Sector Consortia" program in the direction of PCS II "Industry-Science Technology Consortium". The result of the consortium's activities will be:

- - commercialization and introduction of new technology into industrial use.

- - strengthening ties between members of the consortium, which will continue in the form of joint research and development and a joint technological chain.

- 1.1.3 Project coverage and scope

The main directions in the production of shunt reactors are improving the quality of products, using advanced technology for their production, reducing the cost due to the use of new materials and efficient technologies in the development with the use of modern technological equipment, possibly low energy losses during their operation in the network. Materials saving and reducing losses are especially important in electrical networks of 500 kV, in which a significant part of the materials is consumed and a significant part of the energy losses of the entire reactor park occurs.

The production program of the planned facility is shown in Table 2.1.

Results (expected) from the implementation of the Project:

1) a set of drawings and technological documentation issued for production for the manufacture of a shunt reactor and an integrated monitoring system;

2) a ready-made shunt reactor POM-60000/525 with an integrated monitoring system;

3) commercialization of the technology by selling a shunt reactor.

Table 2.1

| № | Task name | Duration | Start | Ending | The end tangible result of the task | Month |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | January | February | March | April | May | June | July | August | September | October | November |
| 1 | Electromagnetic calculations of a reactor with a monitoring system | 3 months | Jan 1 | March 31 | Approved electromagnetic settlement note for the reactor |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Development of design documentation | 3 months | Jan 1 | March 31 | Approved package of design documentation for the reactor and monitoring system, transferred to production |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Purchase of materials, components and equipment | 3 months | Jan 1 | March 31 | Approved contracts for the purchase of materials, components and equipment |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Conducting training for specialists in the basics of Industry 4.0 | 1 month | Apr 1 | May 1 | Certification |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Manufacturing of the magnetic system of the reactor, | 3 months | Apr 1 | June 30 | Finished magnetic system of the reactor |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Reactor winding manufacturing | 3 months | Apr 1 | June 30 | Ready winding of the reactor |  |  |  |  |  |  |  |  |  |  |  |
| 7 | Manufacturing of the main insulation of the reactor | 3 months | Apr 1 | June 30 | Finished main insulation of the reactor |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Reactor tank metal structure fabrication | 3 months | Apr 1 | June 30 | Ready-made metal structure of the reactor tank |  |  |  |  |  |  |  |  |  |  |  |
| 9 | Manufacturing of a monitoring system cabinet | 3 months | Apr 1 | June 30 | Ready-made monitoring system cabinet |  |  |  |  |  |  |  |  |  |  |  |
| 10 | Assembly of the active part of the reactor | 3 months | July 1 | Sep 30 | Ready active part of the reactor |  |  |  |  |  |  |  |  |  |  |  |
| 11 | Reactor monitoring system assembly | 3 months | July 1 | Sep 30 | Complete reactor monitoring system |  |  |  |  |  |  |  |  |  |  |  |
| 12 | Assembly of the reactor tank together with the active part | 3 months | July 1 | Sep 30 | The finished reactor together with the active part |  |  |  |  |  |  |  |  |  |  |  |
| 13 | Reactor acceptance tests | 2 months | Oct 1 | Nov 30 | Reactor acceptance test report |  |  |  |  |  |  |  |  |  |  |  |
| 14 | Routine tests of the reactor monitoring system | 2 months | Oct 1 | Nov 30 | Monitoring system acceptance test report |  |  |  |  |  |  |  |  |  |  |  |
| 15 | Development and implementation of an enterprise automation / digitalization strategy | 11 months | Jan 1 | Nov 30 | Automated manufacturing process |  |  |  |  |  |  |  |  |  |  |  |
| 16 | Registration and obtaining of intellectual property (patents) | 11 months | Jan 1 | Nov 30 | Patent for invention |  |  |  |  |  |  |  |  |  |  |  |
| 17 | Commercialization of a reactor with an integrated monitoring system | 2 months | Oct 1 | Nov 30 | Approved contract for the supply of equipment |  |  |  |  |  |  |  |  |  |  |  |

Description of the technological process for the production and release of finished products

Table 2.2

The work process

| Note : 1 work shift - 8 hours, 40 hour work week, 245 work shifts per year; |  |
| --- | --- |
| The work process | Number of people involved |
| 1) 10 full-time employees for a subproject | The rest are non-staff workers hired for a subproject implementation |
| 2) Planned production capacity | By Order |

Table 2.3

Technological process (stages of production)

| № | Process name | Process description | Equipment used |
| --- | --- | --- | --- |
| 1 | Development of design and technological documentation | Shunt Reactor Modeling; Performing electromagnetic and mechanical calculations; Execution of drawings according to USDD (Unified system for design documentation) and development of technological documentation; Issuance for production of a set of drawings and technological documentation for the manufacture of a shunt reactor. | - |
| 2 | Development of technological documentation | Development of technological instructions for metalworking; Development of technological instructions for welding; Development of technological instructions for painting; Development of technological instructions for reeling the windings of a shunt reactor; Development of technological instructions for the manufacture of electrical insulating parts; Development of technological instructions for assembling a shunt reactor; Development of norms for the consumption of materials for the shunt reactor. | - |
| 3 | Development of technological equipment and devices | Development of drawings of technological equipment and devices | - |
| 4 | Manufacturing of a shunt reactor | Manufacturing of shunt reactor parts, Manufacturing of technological equipment and devices, Welding and assembly works for the fabrication of the shunt reactor metal structure, Surface preparation and painting of metal structures, Winding and crimping windings, Assembly of the active part, Shunt reactor assembly , Manufacturing of electrical insulating parts. | Plasma cutting equipment "CRM" made in Italy; Laser cutting equipment " Bystronic "; Radial drilling machines "KNUTH" made in Germany; Shot-blasting equipment "Airblast"; Winding machines "LAE" made in Italy, mixing table "HIKON" made in Croatia; Drying oven in solvent vapors " Shenyang Youbang Vacuum Equipment " - made in China; Oil degassing plant "Micafluid" made in Switzerland. |
| 5 | Acceptance tests | Insulation resistance measurement, Checking the circuit and group of connecting the windings, Measurement of ohmic resistance of windings to direct current, Testing the dielectric strength of insulations by voltage applied from an external source of industrial frequency 50 Hz, Testing the dielectric strength of insulation with a voltage of increased frequency (the value of the applied voltage at a frequency of 100 Hz = 2Un), Measurement of no-load current, Measurement of no-load losses, Measurement of voltage and short-circuit losses. | - Test station, made in Italy; - WR 14R winding resistance meter ; - Three-phase transformer ratio meter TR-MARK IIIR; - Capacitive voltage divider DIV CC-1-100; - Megaohmmeter C.A. 6547-1. |
| 6 | Delivery of finished products to the warehouse and shipment | -Receiving the shunt reactor to the warehouse; -Order of a railway platform; -Organization of loading the reactor onto the platform; -Organization of lining welding according to the CDD (Chief Designer Department) drawing; - Reactor piping (fastening to the platform) in accordance with the drawings of the CDD; - Registration of shipping documents according to the instructions; - Transfer of the reactor at the station to the carrier; - Notification of the customer about the shipment of the reactor | Hydraulic jacks with lifting capacity up to 30 tons. |
| 7 | Installation supervision and assembly work. |  | - |

Table 2.4

Equipment used

| Name | Registration number | Estimated Cost |  | Purchase method | Purchase term (1, 2, 3 or 4 quarter) | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | per unit | Sum |  |  |  |
| Equipment |  |  |  |  |  |  |
| Monitoring system components | 6587 / G / SH-01 |  | 125 41 000 | FTA | 1-2 quarter |  |
| 1) Transport X2 - analyzer of dissolved gases in oil and moisture |  | 29,500,000 | 29,500,000 |  |  | For the analysis of gases and moisture dissolved in oil, provides accurate information about the state of the shunt reactor |
| 2) Ultrasonic flaw detector |  | 8,500,000 | 8,500,000 |  |  | For ultrasonic testing and measurement of the condition of the shunt reactor tank |
| 3) S179A-21 fiber-optic cable splicer complete with cleaver |  | 3,125,000 | 3,125,000 |  |  | To build fiber-optic monitoring system circuits |
| TRAX 220 substation equipment comprehensive diagnostics system | 6587 / G / NCB-01 | 47,000,000 | 47,000,000 | NCB/ FTA | 1-2 quarter | For measuring and testing the electrical parameters of a shunt reactor |
| Component parts for the manufacture of winding | 6587 / G / NCB-02 |  | 175,769,304 | NCB/ FTA | 1-2 quarter |  |
| 1) Drawing machine with options (8-16mm) TLJ350A |  | 48 593 820 | 48 593 820 |  |  | For processing copper rod with pressure to achieve the required wire cross-section |
| 2) Chiller for drawing machine CK-1008AB |  | 46514940 | 46514940 |  |  | For cooling and heating the drawing machine coolant in order to maintain optimal working conditions |
| 3) WLZB-3-12X 12-way insulation paper braiding machine |  | 40 798 020 | 40 798 020 |  |  | For applying paper insulation to a copper conductor, the ability to wind 12 layers of paper at the same time, depending on the thickness of the insulation |
| 4) 4-way insulation paper braiding machine WLZB-1-4X |  | 39 862 524 | 39 862 524 |  |  | For applying paper insulation to a copper conductor, the ability to wind 4 layers of paper at the same time, depending on the thickness of the insulation |
| Active part assembly platform | 6587 / G / SH-02 | 22 347 960 | 22 347 960 | FTA | 1-2 quarter | For landing the winding on the core of the magnetic core of the shunt reactor |
| Portable monitoring system | 6587 / G / SH-03 | 40,000,000 | 40,000,000 | FTA | 1-2 quarter | For measuring current readings, oil temperature, vibration, grounding shunt reactor |
| Component parts for the assembly of the active part | 6587 / G / SH-04 |  | 20 802 700 | FTA | 1-2 quarter |  |
| 1) Installation of contact soldering УКП-1101 |  | 13,295,000 | 13,295,000 |  |  | For connecting copper taps of the active part by soldering |
| 2) Installation for resistance welding |  | 7,507,700 | 7,507,700 |  |  | For connection of copper bends of the active part by resistance welding |
| Pneumatic packaging tool Columbia ST-POLI HT | 6587 / G / SH-05 | 1 958 668 | 1 958 668 | FTA | 1-2 quarter | For bandaging the active part for the purpose of fastening the structure |
| MLOG Control Monitor | 6587 / G / SH-06 | 1 909 639 | 1 909 639 | FTA | 1-2 quarter | For monitoring the state of the shunt reactor during transportation |
| Laboratory and consumables |  |  |  |  |  |  |
| Black metal rolling | 6587 / G / SH-07 |  | 10 385 411 | FTA | 1-2 quarter | For the manufacture of the metal structure of the shunt reactor tank |
| Electrotechnical cardboard | 6587 / G / SH-08 |  | 15 201 810 | FTA | 1-2 quarter | For the manufacture of insulating parts for the active part of a shunt reactor |
| Reactor core | 6587 / G / NCB-03 |  | 48 593 820 | NCB/ FTA | 1-2 quarter | For the manufacture of the core of the reactor |
| Transformer oil NITRO | 6587 / G / SH-09 |  | 13,556,448 | FTA | 1-2 quarter | For isolation of the active part, as well as cooling the shunt reactor |
| Bushing ВН 550kV | 6587 / G / SH-10 | 25,050,010 | 25,050,010 | FTA | 1-2 quarter | To inject the HV voltage inside the shunt reactor tank |
| Bushing ВН0 35kV: | 6587 / G / SH-11 | 1 978 704 | 3 957 408 | FTA | 1st quarter | To inject the neutral voltage of the HV inside the tank of the shunt reactor |
| Shunt reactor shielding materials | 6587 / G / SH-12 |  | 4 715 211 | FTA | 1-2 quarter |  |
| Air dryer MTraB |  | 1,549,023 | 1,549,023 |  |  | To protect the transformer oil from moisture |
| Gas relay Msafe-MBR80-6 / 4 |  | 800 733 | 800 733 |  |  | To protect the shunt reactor from all types of internal damage to the shunt reactor, accompanied by gas evolution |
| Oil temperature indicator MT-ST160WR / TT / 4/8 |  | 493,903 | 493,903 |  |  | For monitoring the temperature of transformer oil and protecting the shunt reactor from overheating |
| Temperature indicator winding MT-ST160WR / TT / 4/8 |  | 415,227 | 415,227 |  |  | For monitoring the winding temperature and protecting the shunt reactor from overheating |
| Pressure relief valve MPreC |  | 568,451 | 568,451 |  |  | To protect the shunt reactor from overpressure inside the tank |
| Gas sampling device Msafe |  | 147,014 | 147,014 |  |  | To take a gas sample for analysis from a gas relay of a shunt reactor |
| Oil gauge MTO-special |  | 740 860 | 740 860 |  |  | To protect the shunt reactor from a critical decrease or excess of the oil level in the expander |
| IP protection and publications |  |  |  |  |  |  |
| Patenting services | 6587 / S / SH-13 |  | 2,758,000 | FTA | 2-3 quarter | To apply for a patent for invention of the Republic of Kazakhstan |

The transformer oil used in the planned shunt reactor is environmentally friendly and does not contain PCBs. Transformer oil is biodegradable, non-toxic and does not harm the ozone layer.

Transformer oil meets the requirements:

- Technical Regulations of the Customs Union TR CU 030/2012 "On requirements for lubricants, oils and special fluids"

- GOST R 54331-2011 (IEC 60296: 2003)

- TC 38. 1011025-85 “Transformer oil for hydrocracking HC. Technical conditions".

The chemical composition of transformer oil is shown in Table 1.5.

Table 1.5.

| Component | Share in oil |
| --- | --- |
| 1. Paraffins | 10 -15% |
| 2. Naphthenes or cycloparaffins | 60 - 70% |
| 3. Aromatic hydrocarbons | 15-20% |
| 4. Asphalt-resinous substance | 1-2 |
| 5.Sulfur compounds | <1% |
| 6. Nitrogen compounds | <0.8% |
| 7. Naphthenic acids | <0.02% |
| 8. Antioxidant additive (ionol) | 0.2 - 0.5% |

Technical solutions for the production of a shunt reactor

Reducing no-load losses in the designed reactor is achieved through the use of electrical steel 3409 with a thickness of 0.3 mm, with improved characteristics compared to previously used steels and the implementation of the magnetic system of the core according to the scheme with a full oblique joint. And also due to the use of the Step-Lap magnetic circuit blending scheme, the steel is cut on the TUBOLY ASTRONIC cross cutting line, which cuts the plates of the magnetic systems of the reactors in automatic mode according to special programs with a high degree of accuracy.

With the introduction of the line, it ensures the manufacture of magnetic systems with oblique joints in the batching schemes according to the so-called "Step-lap" scheme, due to this, the quality of the assembly of the magnetic circuit increases, and no-load losses are significantly reduced.

The core and yoke plates are cut so that the longitudinal axis of the plate is parallel to the side edge of the roll strip, i.e. coincided with the direction of the rolled steel. With oblique joints in the corners of the magnetic system, additional losses occur, which are less than with straight joints. In this case, the zone of non-coincidence of the direction of the induction lines with the direction of the rolling is limited by a smaller volume of steel adjacent to the joint of the plates. Directly in the junction zone of the laminated magnetic system, an increase in induction occurs and a part of the induction lines from one plate to another passes perpendicular to the surface of the plates. As a result, additional losses occur directly in the joint zone, which are determined by the total joint surface and specific losses per 1 m 2 of the surface.

New designs of magnetic systems are characterized by the use of oblique joints of plates in the corners of the system, tie rods and yokes with circular glass bands instead of through studs in old designs and a multi-stage shape of the yoke section in flat magnetic systems.

Isolation of shunt reactors is a complex system consisting of elements and assemblies that are different both in meaning and design. Insulation during operation is exposed to an unlimited long-term effect of operating voltage and short-term overvoltages: thunderstorm (pulses, duration from units to tens of microseconds), switching (pulses with high attenuation, duration up to several thousand microseconds) and quasi-stationary. Coordination of the internal insulation of the reactor requires the provision of dielectric strength for all these actions.

The main directions of development in the production of insulating materials are currently increasing heat resistance and mechanical strength.

Recently, at the enterprises engaged in the production of shunting, the technology of manufacturing insulating cylinders from thick-sheet hot-pressed cardboard is being introduced instead of paper-bakelite ones. Such cylinders are used in the designed reactor.

This allows, due to the better electrical and mechanical properties of hot-pressed cardboard, to improve the quality of the reactor, as well as to significantly reduce the labor intensity in the production of insulating cylinders.

Hot-pressed cardboard is also used for the manufacture of a large number of insulating parts.

Quantitative characteristics of cardboard:

Heat resistance class -A (105 °);

Overall dimensions: from 1100 mm to 6000 mm, which allows cylinders with a diameter of up to 1.5 m, which greatly simplifies the manufacture of these cylinders

Thickness from 1 to 8 mm.

These cylinders are made of grades A, B, depending on the voltage class.

The blanking of walls and parts at the installation ensures maximum speed, cleanliness of processing and waste-free production. The production of the walls of a rectangular tank is carried out simultaneously and in parallel with the welding of all elements of the tank. Finished walls are welded in special stocks with minimal time and labor input.

Parallel manufacturing of the main assembly elements of the tank, maximum use of new high-performance welding and blanking equipment are the main advantages of the design and technology.

With this technology, the production time of the tanks is reduced by 2-2.5 times. The advantages of a rectangular tank are: Manufacturability of construction due to the possibility of simultaneous production of walls, i.e. production of 4 walls, lid and bottom; production time, it takes less time; simplification of the production of design documentation, drawings of a separate wall are made, and not a general picture.

The use of purchased lugs "Klauke" in comparison with the ones manufactured at the enterprise itself allows to reduce the labor intensity when connecting the wire and the lug. In this case, there is no need for soldering, it is replaced by crimping the wire, this simplifies and speeds up the technological process by several times.

From the point of view of environmental friendliness, the use of Klauke tag terminals is more favorable because there is no need for soldering (no harmfulness), tinning also disappears. When utilizing them, no harmful technological processes are used.

So, the following technical solutions are used in the designed shunt reactor:

1.  The use of electrical steel grade 3409 with a thickness of 0.3 mm with improved performance.

2.  Implementation of the magnetic system of the core according to the "Step-lap" scheme.

3.  Use of insulating cylinders made of electrical insulating thick-sheet hot-pressed cardboard.

4.  The tank is rectangular.

- 1.2     LEGISLATION AND ADMINISTRATION

- 1.2.1 World Bank Environmental Conservation Policy

The following are the rules of the safeguard policy for the Subprojects of the environmental safety category B.

OP/BP 4.01 (Conducting an Environmental Assessment). According to the World Bank's consultation and disclosure guidelines, an overall Environmental Assessment Framework should be prepared prior to undertaking such an assessment. For Sub-projects for which an application for funding has been submitted, which are categorized as B, an (Environment Impact Assessment) EIA and an EMP (Environmental Management Plan) should be prepared.

OP 17.50 (Disclosure Policy) is applied based on the Framework Document, the results of the EIA and the EMPs for the sub-projects for which funding has been applied for.

OP is a technical reference examples of good international industry practice both in general and related to specific industries.

- 1.2.2 National legislation

| Normative legal act (NLA) name | Date and registration number |
| --- | --- |
| Environmental Code of the Republic of Kazakhstan | new |
| Instructions for assessing the impact of planned economic and other activities on the environment during the development of preplanned, planned, pre-project and project documentation | renew |

Normative legal acts regulating the protection of atmospheric air

| NLA title | Date and registration number |
| --- | --- |
| Methodology for determining the standards for emissions into the environment | Appendix to the order of the Minister of Environmental Protection of the Republic of Kazakhstan dated April 16, 2012 No.110-ө |
| Hygienic standards for atmospheric air in urban and rural areas | Approved by order of the Minister of National Economy of the Republic of Kazakhstan dated February 28, 2015 No. 168. Registered with the Ministry of Justice of the Republic of Kazakhstan on May 13, 2015 No. 11036 |
| List of pollutants and waste types for which emission standards are established | Approved by order of the Minister of Energy of the Republic of Kazakhstan dated January 21, 2015 No. 26 |

- 1.3     STATUS OF PROJECT DOCUMENTATION AND AVAILABILITY OF PERMITS

- 1.3.1 Ownership of land or property

Production capacity created on the basis of “Asia Trafo” LLP (“Asia Trafo”) in the Republic of Kazakhstan, Shymkent city, Karatau area, residential area Tassay, Building 1196. The plant for the production of power transformers is located in the north-eastern part of Shymkent in the “Tassay” industrial zone.

The enterprise includes a complex of necessary buildings, structures and premises for industrial and domestic purposes, located on a land plot with an area of 12.0 hectares (cadastral number 19-309-227-1196). The site from the north-west and south-west of the plant borders on the enterprises of the Industrial zone "Tassay", from the north-east on the residential area Zhagalbayly, from the south-east on the highway Zhibek Zholy.

The nearest residential development from the emission sources is located:

- from the southeast - at a distance of 220 m;

- from the north-east - at a distance of 200 m.

There is an act for the right of temporary long-term lease for the land plot No. 400726.

The cadastral number of the plot is 19-309-227-1196.

Purpose of the site: a plant for the production of power transformers of voltage class 110, 220 and 500 kV. The plant consists of complexes of production workshops, an administrative and communal building, a pumping station with a boiler room, dormitories and warehouses.

The planned works within the framework of the implementation of the subproject are included in the existing EIA and the corresponding permit for emissions for these types of works is available.

1.4     Document type or permission

“Asia Trafo” LLP (“Asia Trafo”) emits pollutants into the environment in accordance with the Permit for emissions into the environment for facilities of II and III categories No. KZ49VCZ00710810 dated 23.10.2020.

The enterprise is allowed to produce emissions of pollutants in volumes not exceeding 47.622091 tons / year.

The Permit is valid until December 31, 2029.

- 2        ENVIRONMENTAL DESCRIPTION (BASE LINE STATUS)

- 2.1     General description of the environment at the project site

Production capacity created on the basis of “Asia Trafo” LLP (“Asia Trafo”) in the Republic of Kazakhstan, Shymkent city, Karatau area, residential area Tassay, Building 1196. The plant for the production of power transformers is located in the north-eastern part of Shymkent in the “Tassay” industrial zone.

The enterprise includes a complex of necessary buildings, structures and premises for industrial and domestic purposes, located on a land plot with an area of 12.0 hectares (cadastral number 19-309-227-1196). The site from the north-west and south-west of the plant borders on the enterprises of the Industrial zone "Tassay", from the north-east on the residential area Zhagalbayly, from the south-east on the highway Zhibek Zholy.

The nearest residential development from the emission sources is located:

- from the southeast - at a distance of 220 m;

- from the north-east - at a distance of 200 m.

- 2.2     Climate

The climate of Shymkent is sharply continental with large daily and annual temperature fluctuations.

Cloudy weather for almost the entire year causes a large amount of solar radiation. The duration of the sunshine is 2600-2800 hours. The maximum duration falls on July, varying within 370-420 hours . The number of cloudy days is small and amounts to 40-44 hours a year. The largest number of such days occurs in December and January (8-10 hours per month).

The average annual air temperature varies within the range of +12.4 - + 14 ° С. The absolute maximum temperatures are + 45 ° С.

The coldest month is January, the hottest is July. When cold air masses invade, the air temperature drops sharply. The absolute average long term minimum is - 38 ° С. In winter, weather changes are often observed , thaws last up to 20-30 days, while the air temperature rises to + 15 ° - + 20 ° С. The average long-term values ​​of the moisture deficit are 10.8-12.4 mb.

The distribution of precipitation within the year is very uneven. The greatest amount of precipitation falls in the winter and autumn-spring periods. There is almost no precipitation in summer. The total amount of mixed precipitation per year is 216.86 mm. Such character races - EFINITIONS precipitation throughout the year, when the maximum of their accounts for the period of low temperatures, favors the groundwater recharge. The snow cover is insignificant and unstable. Its average thickness ranges from 9 to 28 cm.

Strong winds prevail in the spring-summer period, the maximum wind speed reaches 20-23 m / s. The prevailing wind direction is from the north, northeast. Winds contribute to more intense evaporation from the surface of water bodies and soil.

It was noted above that the winds of the eastern directions (NE, E, SE) prevail.

Table 1.

Meteorological characteristics of the area where the enterprise is located

| Hаименование характеристик | The magnitude |
| --- | --- |
| Coefficient depending on atmospheric stratification, А | 200 |
| Terrain coefficient of the city | 1.00 |
| Average maximum outdoor temperature of the hottest month of the year, degrees C | 30.4 |
| Average temperature of the outside air of the coldest month (for boiler houses operating according to the heating schedule), degrees С | -0.4 |
| Average annual wind rose,% |  |
| N | 7.0 |
| NE | 11.0 |
| E | 22.0 |
| SE | 21.0 |
| S | 8.0 |
| SW | 12.0 |
| W | 10.0 |
| NW | 9.0 |
| Average annual wind speed, m/s | 2.4 |
| Wind speed (according to long-term average data), the frequency of exceeding which is 5%, m/s | 9.0 |

The value of the coefficient A (coefficient depending on the temperature stratification of the atmosphere), corresponding to unfavorable meteorological conditions, at which the concentration of harmful substances in the atmospheric air is maximum, is taken equal to 200.

The calculated climatological characteristics of the region are presented in Table 1. (according to the form presented in [3], it is displayed automatically by the "ЭРА" program).

- 2.3     Geomorphology

The territory under consideration is located on the surface of the earth, hilly-ridged with a general slope from west to east.

The absolute marks of the territory are 550.35-600.00 m.

- 2.4     Surface water

There are no surface water bodies and water courses in the area of ​​the enterprise. There is no negative impact on surface waters. Wastewater will not be discharged into surface water. The construction project provides for a drainage system into the existing sewer network with a diameter of 300 mm.

- 2.5     The groundwater

Groundwater, according to drilling data for April-May 2016 to a depth of 20.0 m from the earth's surface, has not been discovered. There is no negative impact on groundwater.

- 2.6     Vegetation, fauna

The projected production is located on an initially anthropogenically disturbed territory. There are no flora and fauna on the site.

- 2.7      Land resources

The projected production is located in the existing premises and additional land acquisition is not required.

- 2.8     Socio-cultural environment

Any economic activity can have the consequences of a change in the social conditions of the region, both in the direction of increasing the benefits and benefits of the local population in the field of economy, education, health care, and in the direction of the deterioration of the social and environmental situation as a result of unforeseen consequences.

The production of products will directly or indirectly affect the following aspects affecting the interests of the population living in the area of ​​influence of the projected activities:

• traditional and legal rights to use land resources;

• use of the territory by persons not permanently residing on it;

• the nature of the use of natural resources;

• the state of health of the population.

There are no archaeological and cultural-historical objects on the site of the enterprise, which can be adversely affected during the operation of the enterprise.

- 3        DETERMINING POTENTIAL IMPACT

Atmospheric air. According to clause 17 of the "Instructions for Conducting Environmental Impact Assessment", approved by the order of the Minister of Environmental Protection of the Republic of Kazakhstan dated June 28, 2007 No. 204-п at the Pre-EIA stage, the types and intensity of the impact of the planned economic activity are determined by analogous projects. To assess the impact of the projected production on the atmospheric air, the data on the impact on the atmospheric air of the Workshop for the production of 35-110 kV transformers of the Kentau Transformer Plant JSC with a similar production technology were accepted.

Qualitative and quantitative parameters of emission standards adopted according Project maximum allowable emissions of harmful substances into the atmosphere for «Asia Trafo» LLP ("Asia Trafo") having a positive conclusion by the state ecological expertise № KZ49VCZ007110810 dated 23.10.2020.

Maximum permissible concentrations (MPC) of pollutants in the ambient air of populated areas are adopted in accordance with the "Hygienic standards for atmospheric air in urban and rural settlements", approved by the order No. 168 of the Minister of National Economy of the Republic of Kazakhstan dated February 28, 2015.

Emissions of pollutants for the enterprise as a whole, taking into account the projected production, are shown in Table 3.1.

Table 3.1

| Substance code | Name of substances | Standards for emissions of pollutants into the atmosphere * |  |
| --- | --- | --- | --- |
|  |  | g / s | t / year |
| 101 | Aluminum oxide | 0.00000833 | 0.00006 |
| 0123 | Iron (II, III) oxides | 0.479815 | 6.49541 |
| 0143 | Manganese and its compounds in terms of manganese (IV) oxide / (327) | 0.0178586 | 0.219505 |
| 0301 | Nitrogen (IV) dioxide (Nitrogen dioxide) (4) | 0.896097 | 11.56076 |
| 0304 | Nitrogen (II) oxide (Nitrogen oxide) (6) | 0.145599 | 1,87771 |
| 0330 | Sulfur dioxide (sulfur dioxide, sulfur dioxide, sulfur (IV) oxide) (516) | 0.024332 | 0.26832 |
| 0337 | Carbon monoxide (Carbon monoxide, Carbon monoxide) (584) | 1.45455 | 16.4556 |
| 0616 | Dimethylbenzene (mixture of o-, m-, p-isomers) (203) | 0.322 | 3.018 |
| 0621 | Methylbenzene (349) | 0.0694 | 0.604 |
| 1042 | Butan-1-ol (Butyl alcohol) (102) | 0.0602 | 0.5834 |
| 1061 | Ethanol (ethyl alcohol) (667) | 0.0139 | 0.1 |
| 1119 | 2-Ethoxyethanol (Ethylene glycol ethyl ether, Ethyl cellosolve) (1497 *) | 0.0852 | 0.693 |
| 1210 | Butyl Acetate (Acetic Acid Butyl Ester) (110) | 0.0139 | 0.1 |
| 1213 | Etenyl acetate (Vinyl acetate, Acetic acid vinyl ester) (670) | 0.002084 | 0.015 |
| 1401 | Propane-2-one (Acetone) (470) | 0.1002 | 1.481 |
| 2735 | Mineral petroleum oil (spindle, machine, cylinder, etc.) (716 *) | 0.076363 | 0.32828 |
| 2750 | Solvent naphtha (1149 *) | 0.1451 | 1.00222 |
| 2902 | Suspended particles (116) | 0.21944 | 1.65497 |
| 2908 | Inorganic dust containing silicon dioxide in%: 70-20 cement, cement production dust - clay, shale, blast-furnace slag, sand, clinker, ash, silica, ash from Kazakh coal deposits) (494) | 0.0007761 | 0.00559 |
| 2930 | Abrasive dust (White corundum, Monocorundum) (1027 *) | 0.026 | 0.0936 |
| 2962 | Dust paper (1034 *) | 0.08558 | 1.065666 |
|  | TOTAL: | 4.23840303 | 47.622091 |

Qualitative and quantitative parameters of existing enterprise emission sources according to accepted standards of maximum permissible emission (MPE) of harmful substances into the atmosphere for “Asia Trafo” LLP (“Asia Trafo”) having a positive conclusion by the state ecological expertise № KZ49VCZ007110810 dated 23.10.2020.

As follows from the abovementioned data, emissions of pollutants into the atmosphere for the enterprise as a whole will not change with the commissioning of a new production, since the projected production will be located on the site of one of the workshops while maintaining the quantity and quality of emissions.

According to clause 18 of the "Instructions for Conducting Environmental Impact Assessment", approved by the order of the Minister of Environmental Protection of the Republic of Kazakhstan dated June 28, 2007 No. 204-п at the Pre-EIA stage, calculations of the level of pollution of individual components of the environment are not required.

The significance of the impact of the projected production with existing sources of emission is determined based on the calculations performed as part of the draft maximum allowable emissions of harmful substances into the atmosphere for “Asia Trafo” LLP (“Asia Trafo”) having a positive conclusion by the state ecological expertise № KZ49VCZ007110810 dated 23.10.2020.

The calculation of atmospheric pollution was carried out in accordance with the "Methodology for calculating the concentrations of harmful substances in the emissions of enterprises", using the unified program for calculating atmospheric pollution UPCAP ЭРА v 2.0.

Noise impact. Vibration. The main source of noise and vibration in production is technological and engineering equipment. The characteristics of the technological and engineering equipment for the production of shunt reactors are contained in their technical documentation and comply with hygienic requirements. The distance from the border of the enterprise to residential areas is more than 100 m. Protection against noise and vibration is provided by the design solutions of the equipment used. A factor in the increase in noise and vibration levels is the mechanical wear of technological equipment and its assemblies, therefore, to prevent possible exceeding of noise and vibration levels, the following measures should be taken:

- control measurements of noise and vibration at the workplaces of drivers and operators, which are carried out by a specialized organization at least once a year;

- if the levels of noise and vibration are exceeded, a control survey is carried out in order to establish the cause and take measures for the replacement or repair of units;

- periodic check of equipment, machines and mechanisms for the presence and serviceability of sound-absorbing casings, facings and enclosing structures, vibration isolation of control handles, steps, seats, platforms of operating machines.

The sanitary protection zone provided for by the subproject and confirmed by the results of calculations of the dispersion of harmful emissions into the atmosphere is sufficient to exclude hygienically significant acoustic impacts on the adjacent territories. The planning and technical solutions laid down in the subproject meet the requirements of noise protection. The noise of the sources incorporated in the project can be taken for the remote control.

Water resources. Water consumption and wastewater disposal of the projected production is a secondary factor of its impact on the environment, since it is carried out by connecting it to the centralized networks of Shymkent city. Water is not used in the shunt reactor production technology. The source of household and industrial water supply of the enterprise will be the existing networks of the enterprise connected to the city water supply networks. The estimated volume of water demand for household needs will be 100 liters/hour, 2400 liters/day. Hot water for sanitary needs will be used in a volume of 500 liters/day. The production of shunt reactors does not have industrial waste water.

Household sewerage is gravity, it is provided for the drainage of wastewater from sanitary appliances with a tie-in into the existing intra-workshop networks of the same name. Wastewater discharge into the environment is not provided for by the project.

Waste from production and consumption. Waste from the projected production is represented by waste of consumption and production waste.

Waste consumption is represented by municipal solid waste received as a result of the life of the working personnel. Municipal solid waste (MSW) at the enterprise is generated as a result of non-production activities of production personnel, as well as during cleaning of premises and territory. MSW generated at the enterprise is accumulated in a container with a volume of 1.2 m3. Further, as it accumulates, municipal solid waste is transported to the solid waste landfill. According to the Waste Classifier, MSW is classified as a green waste level with the code GO060.

The enterprise has a unified waste management system. The introduction of a new production technology for the reactor is not associated with an increase in the number of employees and will not lead to an increase in the generation of municipal waste.

Industrial waste is represented by metal waste. The specific rate of waste generation will be 214 kg/ton of the metal used. Waste metal is sorted according to the type of metal and stored in special containers with a capacity of 1.2 m3. As it accumulates, but at least once a month, metal waste is transported under a contract to specialized enterprises for disposal.

Waste metals belong to the green hazard level and have codes established in accordance with the "Waste Classifier", approved by the order of the Minister of Environmental Protection of the Republic of Kazakhstan dated May 31, 2007 N 169-п:

- waste and scrap of ferrous metals - GA090;

- waste and scrap of copper - GA120.

The project does not provide for its own landfills and places for long-term waste disposal.

Land resources, soil, flora and fauna. For the realization of planned activity the existing premises will be used, which are owned by “Asia Trafo” LLP (“Asia Trafo”) in the Republic of Kazakhstan, Shymkent Karatau area, residential area Tassay, building 1196.

The projected production does not require additional land allocation. The purpose of the land plot does not change. Construction works are not provided for by the project, the fertile soil layer is not disturbed.

Considering that the projected production is located on an initially anthropogenically disturbed territory, its impact on flora and fauna is excluded.

Measures to reduce environmental risk. The most important role in ensuring the safety of workers and the local population and the protection of the environment is played by a system of rules, regulations, instructions and standards, the observance of which is mandatory by managers and all employees.

Recommendations for the prevention of emergency situations include the following measures:

- resolve issues of notifying employees, gathering management personnel, organizing communications at any time of the day;

- appoint persons responsible for the measures in the event of an emergency;

- create and equip civil defense units and train personnel;

- strengthen the security of the facility;

- prepare a place for providing medical assistance to victims;

- plan evacuation measures.

Safety and fire prevention measures. Only persons trained according to a special program, who have passed exams and received the appropriate certificate for safety, industrial sanitation and fire safety, are allowed to work on operation and maintenance.

- 4.1. IMPACT MITIGATION AND MONIToring

| - Activity | - Expected environmental impact | - Proposed mitigation measure | - Responsibility for implementing mitigation measures | - Period of implementation of mitigation measures |
| --- | --- | --- | --- | --- |
| Working on a paper milling machine | Emissions to the atmosphere of paper dust | Equipped with a 99% dry bag filter. | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | Before putting production into operation |
| Work on the machine for semi-automatic welding of steel in carbon dioxide | Organization of emission in order to improve the dispersion of pollutants | Equipment of the apparatus with exhaust units of the SovPlim brand. | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | Before putting production into operation |
| Work on metalworking machines | Organization of emission in order to improve the dispersion of pollutants | Equipment of the apparatus with exhaust units of the SovPlim brand. | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | Before putting production into operation |
| Painting line | Emissions to the atmosphere of paint vapors | Equipping with hydrofilters and filters | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | Before putting production into operation |
| Production and consumption waste management | Reducing the volume of landfills | Separate waste collection and sorting them according to their components in order to utilize individual waste components. Waste generation and disposal records will be maintained and the laboratory will conduct regular checks on the records. Waste storage will be carried out in a specially designated place with garbage disposal in accordance with sanitary standards. | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | Constantly |
| Filling the reactor tank with oil | Prevention of environmental pollution by persistent organic pollutants | Use in the manufacture of shunt reactors of oil that does not contain polychlorinated biphenyls | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | Constantly |
| Equipping the premises with equipment and operation | Occupational health and safety risks | In the course of work, all employees wear overalls and personal protective equipment: a cotton suit, goggles, leather boots with a metal toe, combined gloves. Keeping training logs (introductory, primary, electrical safety, fire safety). Use of rational work and rest regimes for workers. | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | Everyday Every quarter Everyday |
| Equipping the premises with equipment and operation | Danger of electric shock | - Overload protection in equipment design - Complete blocking of power supply when the case is open. -in the event of a power outage, the starting devices are moved to the "Stop" position until the malfunctions are eliminated by workers who have a permit for electrical safety. - control of electrical circuits of automation in accordance with the Equipment Passport by a specialized organization | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | Constantly |
|  | Fire safety | Check for the presence of a fire extinguisher, which is used to extinguish burning materials of all types - solid flammable materials, flammable liquids and electrical installations up to 1000 volts. All members of the performer group subproject undergo regular fire safety briefing. Keeping a log of safety briefing. Periodic checking the condition of fire-fighting equipment. The emergency plan will be periodically reviewed and updated. Periodic verification of control systems. Regular checks and maintenance of the ventilation system. | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | Constantly |
| Installation supervision and assembly work | Noise / vibration - Compliance with the schedule working time; - Noise levels (in case of complaints) | - At the workplace, an inspection will be carried out for compliance with the noise level (permissible noise level 75-80dB) - Upon receipt of complaints, measures will be taken to eliminate the causes of increased noise. Also, measuring noise levels with a multifunctional environmental meter | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | During assembly operations |
|  | Short-term exposure to dust (suspended particulate matter Single time Maximal Permissible Pollutant Concentration = 0.5 mg/ m3). | Providing workers with respirators. - Use of measuring instruments (when checked by a government agency / licensed laboratory). | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | During assembly operations |
| Installation supervision and assembly work | Quality control | When purchasing equipment and installing it, quality control will be carried out: equipment must have certificates of origin and compliance with all technical standards, have passports, safety and operating instructions. | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | During assembly operations |
| Delivery of finished products to the warehouse and shipment | Safety engineering | Quality control will be carried out in the quality control area with regular checks of records. Separate, selective quality control will be carried out in the laboratories of specialized organizations on a contractual basis. | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | Upon dispatch |
|  | Fire safety | Check for the presence of a fire extinguisher, which is used to extinguish burning materials of all types - solid flammable materials, highly flammable liquids and electrical installations up to 1000 volts. All members of the performer group subproject undergo regular fire safety briefing. Keeping a log of briefing on safety precautions. Periodic inspections of the condition of the fire fighting equipment will be carried out. The emergency plan will be periodically reviewed and updated. Periodic verification of control systems. Regular checks and maintenance of the ventilation system. | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | Upon dispatch |
| Delivery of finished products to the warehouse and shipment | Product quality control | Quality control will be carried out in the quality control area with regular checks of records. Separate, selective quality control will be carried out in the laboratories of specialized organizations on a contractual basis. | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | During the production and delivery of products |
| Emergency requirements | Reduction of emergency situations during project implementation | Stop work immediately if a situation arises that may lead to an accident or accidents, turn off the equipment in use. - In the event of a fire or fire, you must immediately inform the fire department by phone number 101, inform the management and start extinguishing the fire with the available fire extinguishing means. - In case of an accident, the employee must provide first aid to the victim, call an ambulance by dialing 103, inform the administration about the accident, if possible, preserve the situation until the causes of the incident are investigated, if this does not lead to an accident or injury to other people. | Production Director. | Constantly |
| Providing first aid workers | Reducing the consequences of potential safety violations by personnel during production work | The employer is obliged to ensure the constant possibility of providing employees with qualified first aid. First rendering point medical aid with the necessary equipment should be located in a place that is easily accessible from any work area. | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) | Constantly |
| Procedure instructions | Reduction of emergency situations during project implementation | In remote work areas, there should be written instructions on how to deal with injuries and serious illness of workers up to and including transportation of the injured and sick to the appropriate medical institution. | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”), head of the division | Constantly |

- 4.2 MONITORING PLAN

| Which parameter should be monitored? | Where should the monitoring parameter be monitored? | How should the monitoring parameter be monitored (what should be measured and how)? | When should the monitoring parameter be monitored (time and frequency)? | Who should track this parameter (responsibility)? |
| --- | --- | --- | --- | --- |
| Emissions of pollutants into the atmosphere | Stationary emission sources | -Maximum one-time emission (g / s) by the instrumental method or by calculation (for fugitive emission sources). In total, the inventory carried out in 2020 on the territory of the enterprise revealed 31 sources of emissions, including 15 controlled, 16 - fugitive. For controlled sources, quarterly measurements are carried out by an independent accredited laboratory in accordance with the IEC (IEC-industrial environmental control). Monitoring of unorganized sources is carried out according to the actual operating time by the calculation method. - Use of measuring instruments (when checked by a government agency / licensed laboratory). - Carrying out instrumental measurements at organized sources in accordance with the control schedule at the enterprise for compliance with the MPE standards. - Collecting actual data necessary for calculating emissions, carrying out calculations with a frequency of at least once a month. - Conclusion of contracts with an accredited laboratory to control the established emission standards in accordance with the approved environmental control program. - Submission of environmental reports to the authorized environmental protection body. - Control of local exhaust ventilation at the points of possible emissions and leaks of harmful substances at least once a month. - Continuous monitoring of the storage of hazardous substances in closed containers/ packages. | Quarterly at maximum equipment load | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) with the assistance of a certified laboratory |
| Volumes of waste generation, temporary storage and transfer to third parties | At sources of waste generation, in places of their temporary storage | -Accounting for the volume of waste generation. - Accounting for the Frequency of transfer of waste to third-party organizations licensed for the following types of waste: 1. Cinders of welding electrodes; 2. Waste accumulators, lithium batteries; 3. Metal waste from plasma cutting apparatus and shot blasting chambers; 4. Wood and cardboard sawdust; 5. Spent filters from the spray booth; 6. Plastic waste; 7. Container (from enamel, from solvent, from primer, from hardener. From solvent) 8. Hydraulic oil; 9. Waste tires; 10. Departure from cutting tools; 11. Oiled rags; 12. Waste transformer oil; 13. Office equipment, power tools, Cartridges; 14. Paper; 15. Silicone; 16. Glass; 17. Waste of abrasive materials; 18. Lamps (fluorescent, mercury-containing, quartz, energy-saving). -Separate collection of waste and sorting them into components for the purpose of utilization of individual components of waste. -Records of waste generation and disposal will be maintained and regular checks of records will be carried out. -Submission of environmental reports to the authorized environmental protection body. | Constantly | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) with the assistance of a certified laboratory |
| The composition of the used transformer oil | Before the stage of pouring it into reactors | Confirmation of the composition of the applied transformer oil by an accredited laboratory. Check for the presence or absence of polychlorinated biphenyls and maintain appropriate records. | Constantly | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) with the assistance of a certified laboratory |
| Occupational safety and health | During production work | - Determination and provision of the required PPE, which provide the necessary protection for workers at intervals of at least once a month or in accordance with the rules for issuing PPE. 1. Cotton suit set with oil-water-repellent; 2. Boots leather with metal. toe; 3. Tarpaulin mittens; 4. Goggles, light filter; 5. Protective helmet; 6. Apron with bib, fireproof; 7. Summer T-shirts (gray); 8. Headphones or earplugs; 9. Oversleeve; 10. Underwear (in winter); 11. Felt insoles (in winter); 12. Winter socks (in winter); 13. Warm vest; 14. Warm winter hat; 15. Leather shoes (for the summer period); 16. Welding suit set with split; 17. Gauntlet gloves; 18. Welding mask; 19. In winter, the jacket and trousers made of wool; 20. Hardhat liner; 21. ESAB Origo Spec 5 DIN glasses; 22. Aspect shotgun helmet with panoramic views, durable nylon apron; 23. Suit of a shot blaster with gloves; 24. Reusable half mask 3M; 25. Overalls brand "ТАИВЕК" (polyethylene); 26. Respirator; 27. Acid Alkali Resistant gloves; - Continuous monitoring of the use of PPE. - Monitoring the maintenance of PPE, including cleaning when contaminated and replacing it when damaged or worn out, at least once a month. - Training of employees in the rules of proper use of PPE at intervals of at least once every quarter. - Maintaining documentation on the availability of personal protective equipment and the need for training in actions in the event of emergencies. - Control over the frequency of certification of workplaces, which is carried out by specialized organizations with a frequency of at least once in five years. - Mandatory instruction for all new employees on occupational health and safety issues in order to inform them about the most important rules of work in certain work areas, as well as about the available personal protective equipment and prevention of injuries. - Basic briefing on health and safety issues and special classes in areas of work in accordance with production needs in order to inform all workers about the hazards associated with the implementation of specific types of production tasks. - Visually by verifying the practical use of the equipment and checking the existing equipment conformity to the regulations. Training records will be reviewed and improvements made as necessary. | Constantly | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) with the assistance of a certified laboratory |
| Fire safety | During production work | Periodic inspections of the condition of the fire fighting equipment will be carried out. The emergency plan will be periodically reviewed and updated. Training and testing of knowledge on occupational safety and health issues for managers and employees responsible for ensuring safety and security. Keeping an appropriate journal about intruction and training. | Constantly | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) with the assistance of a certified laboratory |
| Inspection of technological equipment | Before starting and during operation of the equipment | Inspection, revision for the serviceability of technological equipment. - Training of personnel in the operation of equipment. Access to the equipment only after testing knowledge and skills. Equipment knowledge test records will be maintained, reviewed and improvements made as necessary. Availability of certificates of conformity for the raw materials and equipment used. | Constantly | Head of Occupational Safety and Health, Responsible for the Environment “Asia Trafo” LLP ( “Asia Trafo”) with the assistance of a certified laboratory |

Public hearings

The  Environmental Management Plan is presented for review in the public domain on the website https://trencontc.kz/en/news/environmental-management-plan-for-the-subproject-development-of-a-500-kv-shunt-reactor-with-an-integrated-monitoring-system-and-implementation-in-production-with-the-purpose-of-commercializing-the-results-of-scientific-and-technical-activities

A public hearing was held, during which representatives of the Technology Customer, as well as local residents, got acquainted with the environmental protection action plan, asked questions, expressed comments and suggestions. The minutes of public hearings dated 12 December, 2021 are attached.